UBio-Alpeta

Real-time event process UNION COMMUNITY

Real-time event transmission method

: Supports two-way communication between server and client using web socket

- What is the web socket? Web Socket is a structure to do an efficient two-way communication  between server and client. It is one of the communication protocol for network and a technical  rule for two-way communication defined by the World Wide Web Consortium (W3C) and the  Internet Engineering Task Force (IETF), the standardization organizations of the Internet, to  define the rules for communication between the server and the client.

How to access to Web Socket

: Access to the Web socket address.

- ws://192.168.0.2:9004/v1/webEntry -> IP address is the server IP installing the Alpeta.

Example of real-time event information data

Event message name: WSCmdTerminalLiveInfo

Received data:

{

msgId: 1102,

result: 0,

body: "[{

"ID":8,

"Status":25169945,

"Type":37

}]"

}

The data including msgId, result, and body(Json) is sent from server to client.

| Name | Value | Description |
| --- | --- | --- |
| msgId | 1102 | The real-time event message ID. |
| result | 0 (Fixed) | Server -> Client (0: Fixed) |
| body |  |  |
| ID | 8 | Terminal ID |
| Status | 25169945 | Terminal Status value (Refer to the table 2 below.) |
| Type | 37 | Terminal type (Refer to the menu of WSCmdTerminalLiveInfo below.) |

The types of real-time event

| Event name | Event message ID | Description |
| --- | --- | --- |
| WSCmdLogoutNotify | 1002 | Logout |
| WSCmdLogAuthNotify | 1100 | Terminal authentication log |
| WSCmdLogEventNotify | 1101 | Terminal event log |
| WSCmdTerminalLiveInfo | 1102 | Terminal status information |
| WSCmdServiceStatusNotify | 1103 | Server status information |
| WSCmdAuthLogImageNotify | 1104 | Terminal authentication log image |
| WSCmdTerminalNoticeSetNotify | 1105 | Notice for terminal |
| WSCmdGroupSyncNotify | 1106 | Change the group information. |
| WSCmdPositionSyncNotify | 1107 | Change the position information. |
| WSCmdTerminalSyncNotify | 1108 | Change the terminal information. |
| WSCmdAccessGroupSyncNotify | 1109 | Change the information of access group. |
| WSCmdTimezoneSyncNotify | 1110 | Change the time-zone information. |
| WSCmdPrivilegeSyncNotify | 1112 | Change the privilege information. |
| WSCmdUserMessageSyncNotify | 1113 | Change the user’s message. |
| WSCmdTerminalLogUploadNotify | 1120 | The data of importing the terminal log. |

WSCmdLogoutNotify (msgId: 1002)

: Sends the logout event from server to web client.

: Received data:

{ msgId: 1002, result: 0, body: "{" UserId ":1000000000000000000," Type ":0}"}

| Name | Value | Description |
| --- | --- | --- |
| msgId | 1002 | Real-time event message ID |
| result | 0 (Fixed) | Server -> Client (0: Fixed) |
| body |  |  |
| UserId | 10000000… | User ID |
| Type | 0 | The types of logout 0: When the control server to ask log-in is different or when requesting the web log-in newly. 1: Request the automatic log-out by initializing since there wasn’t login extension for 30 minutes. 2: When deleting the log-in users |

WSCmdLogAuthNotify (msgId: 1100)

: When the user authenticates at the terminal, an event occurs that sends an  authentication log to the monitoring page

: Received data:

{msgId: 1100, result: 0, body: {"IndexKey":0,"TerminalID":3,"UserID":"","UserID":0, "UserName":"","UniqueID":"","EventTime":"2019-12-20 17:45:13",

"ServerRecordTime":"1970-01-01 00:00:00","AuthType":2,"AuthResult":1,"Func":3,"FuncType":0, "Card":"","UserType":0,"IsPicture":1,"Property":"1000","Latitude":0,"Longitude":0,"GroupName":"", "PositionName":""}}"}

| Name | Value | Description |
| --- | --- | --- |
| msgId | 1100 | Real-time event message ID |
| result | 0 | Server -> Client (0: Fixed) |
| body |  |  |
| IndexKey | 0 | The index for saving DB (0: Fixed) |
| TerminalID | 8 | The registered terminal ID |
| UserID | “1” or “” | User ID (If unregistered, “”) |
| UserName | “Tom” | User name |
| UniqueID | “19-0153” | Unique ID |
| EventTime | “2019-12-20 17:45:13” | Event occurrence time |
| ServerRecordTime | “1970-01-01 00:00:00” | DB saving time (No use) |
| AuthType | 2 | Authentication type 1: FP 1:1 authentication 2: FP 1:N authentication 3: Password authentication 4: Card authentication 5: Face 1:1 authentication 6: Face 1:N authentication 7: Mobile card 8: QR code |

|  |  | 11: Request the authentication type of ID or Unique ID |
| --- | --- | --- |
| AuthResult | 1 | Authentication result 0: Success 1: Fail 2: Not available to access 3: Time out 4: Capture time out 5: 1:N time out 6: Not available for anti-pass back 7: Duress 8: Blacklist |
| Func | 3 | Function key 1: F1 2: F2 3: Access 4: F3 5: F5 TnA 11: Attend 12: Leave 13: In 14: Out 15: Return Meal 21: Menu 1 22: Menu 2 23: **** 24: Menu 3 25: Menu 4 |
| FuncType | 0 | Function type (No use) |
| Card | “A01032” or “” | Authentication card number |
| UserType | 0 | User type (0: Fixed) |
| IsPicture | 1 | The flag to whether there is an image or not |
| Property | “1000” | 1000 (Fixed) |
| Latitude | 0 | The latitude when uploading the authentication log from V-Terminal (Smartphone) / (Not currently available) |
| Longitude | 0 | The longitude when uploading the authentication log from V-Terminal (Smartphone) / (Not currently available) |
| GroupName | “Group name” | Group name |
| PositionName | “Position name” | Position name |

WSCmdLogEventNotify (msgId: 1101)

: When other events such as door opening or fire occur in the terminal : Received data:

{msgId: 1101, result: 0, body:

"{"TerminalID":8,"EventTime":"2019-12-23 10:16:41","Category":2,"Content":131083,"Detail":""}"}

| Name | Value | Description |
| --- | --- | --- |
| msgId | 1101 | Real-time event message ID |
| result | 0 (Fixed) | Server -> Client (0: Fixed) |
| body |  |  |
| TerminalID | 8 | Terminal ID |
| EventTime | 2019-12-23 10:16:41 | Event occurrence time |
| Category | 2 | Event category -> 1: Terminal, 2: Door for access, 3: Emergency situation, 4: External signal, 5: System |
| Content | 131083 | Event category (Refer to the table 1 below. ) |
| Detail |  | Detailed information |

WSCmdTerminalLiveInfo (msgId: 1102)

: Imports the terminal status information periodically. (Every 10 seconds) : Received data:

{ msgId: 1102, result: 0, body: "[{"ID":8, "Status":25169945, "Type":37}]"}

| Name | Value | Description |
| --- | --- | --- |
| msgId | 1102 | Real-time event message ID |
| result | 0 (Fixed) | Server -> Client (0: Fixed) |
| body |  |  |
| ID | 8 | Terminal ID |
| Status | 25169945 | Terminal status value (Refer to the table 2 below.) |
| Type | 37 | Terminal type information 2 : NAC 2500 3 : NAC 3000 4 : NAC 2500 (4MF) 6 : NAC 5000 9 : NAC 1500 18 : T5 19 : T3 20 : T1 22 : T9 23 : FKA2 24 : eNCardi 25 : T2 26 : UBio-X Slim (Nitgen) 30 : AC1100 31 : AC2000 32 : AC2200 33 : AC5000 34 : AC5100 35 : AC7000 36 : UBio-X Pro Lite 37 : UBio-X Pro 38 : AC6000 39 : UBioTablet5 40 : UBio-X Slim (Virdi) |

WSCmdServiceStatusNotify (msgId: 1103)

: Gets periodically the system status information.

: Received data:

{ msgId: 1103, result: 0, body:

"{"DSInfo":{"ServerID":1,"ServerType":1,"ServerMode":1,"t":{"seconds":1577065046,"nanos":632693 900},"DBStat":{"User":{"curCnt":0},"Term":{"curCnt":10},"Auth":{"curCnt":0},"Sys":{"curCnt":0}},"S ystemStat":{"ServerID":1,"totalMemory":7,"usedMemory":"70.00","idleCPU":"54"}},"CSInfo":{"Server ID":11,"totalMemory":7,"usedMemory":"70.00","idleCPU":"54"},"ASInfo":{"ServerID":100,"ServerType ":0,"ServerMode":0,"t":null,"DBStat":null,"SystemStat":{"ServerID":0,"totalMemory":7,"usedMemory": "70.00","idleCPU":"112553.98"}},"FPStatInfo":{"curMinute":37,"fpTotal":0,"fpReqCount":[0,0,0,0,0,0, 0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,1,2, 0],"fpProcessTime":[0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0, 0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,5,7,0]},"TaskInfo":null}"}

(When having the data below)

| Name | Value | Details |
| --- | --- | --- |
| msgId | 1103 | Real-time event message ID |
| result | 0 | Server -> Client (0: Fixed) |
| body |  |  |
| DSInfo |  |  |
| - ServerID | 1 | Server ID |
| - ServerType | 1 |  |
| - ServerMode | 1 |  |
| - t |  |  |
| - seconds | 1576735379 |  |
| - nanos | 154753900 |  |
| - DBStat |  |  |
| - User |  |  |
| - curCnt | 0 | The number of users |
| - Term |  |  |
| - curCnt | 8 | The number of terminals |
| - Auth |  |  |
| - curCnt | 0 |  |
| - Sys |  |  |
| - curCnt | 0 |  |
| - SystemStat |  |  |
| - ServerID | 1 | Server ID |
| - totalMemory | 7 | Total memory capacity |
| - usedMemory | 73.00 | Used memory capacity |
| - idleCPU | 53 | CPU free capacity |
| CSInfo |  |  |
| - ServerID | 11 | Server ID |
| - totalMemory | 7 | Total memory capacity |
| - usedMemory | 73.00 | Used memory capacity |
| - idleCPU | 53 | CPU free capacity |
| ASInfo |  |  |
| - ServerID | 100 | Server ID |
| - ServerType | 0 |  |
| - ServerMode | 0 |  |
| - t | null |  |
| - DBStat | null |  |

| - SystemStat |  |  |
| --- | --- | --- |
| - ServerID | 0 | Server ID |
| - totalMemory | 7 | Total memory capacity |
| - usedMemory | 73.00 | Used memory capacity |
| - idleCPU | 93506.34 | CPU free capacity |
| FPStatInfo |  |  |
| - curMinute | 2 |  |
| - fpTotal | 0 | The number of server FP authentication |
| - fpReqCount | [0,0,0,0,0,0,0…] |  |
| - fpProcessTime | [0,0,0,0,0,0,0…] |  |
| TaskInfo | null |  |

WSCmdAuthLogImageNotify (msgId: 1104)

When authenticating in terminal while the terminal monitoring detail page is opened,  the image taken on the terminal camera is transmitted. (The event doesn’t occur when  the terminal authentication mode is ‘terminal’ mode.)

Received data:

{msgId: 1104, result: 0, body:

"{"header":{"cmdId":1093673220,"uniqueKey":4294967295,"result":0,"DesType":1,"DesID":0, "UserID":0,"brandType":1},"TerminalID":8,"LogIndex":201912000000089,"TerminalLogIndex":0, "EventTime":{"seconds":1577062673},"PictureInfo":{"Type":"jpg","Length":6490,"Data":"/9j/4AAQSk ZJRgABAQAAAQABAAD/2wBDABALDA4MChAODQ4SERATGCgaGBYWGDEjJR0oOjM9PDkzODdASFxOQ ERXRTc4UG1RV19iZ2hnPk1xeXBkeFxlZ2P//2Q==…"}}"

(When having the data below)

| Name | Value | Description |
| --- | --- | --- |
| msgId | 1104 | Real-time event message ID |
| result | 0 | Server -> Client (0: Fixed) |
| body |  |  |
| header |  |  |
| - cmdId | 1093673220 |  |
| - uniqueKey | 42949672… |  |
| - result | 0 |  |
| - DesType | 1 |  |
| - DesID | 0 |  |
| - UserID | 0 |  |
| - brandType | 1 |  |
| TerminalID | 8 | Terminal ID |
| LogIndex | 201912000000089 |  |
| TerminalLogIndex | 0 |  |
| EventTime |  |  |
| - seconds | 1577062673 |  |
| PictureInfo |  |  |
| - Type | jpg |  |
| - Length | 6490 |  |
| - Data | /9j/4AAQSkZ… |  |

WSCmdTerminalNoticeSetNotify (msgId: 1105)

: When sending the notice from the notice management page to terminal : Received data :

{ msgId: 1105, result: 0, body: "{"TerminalID":8}"}

| Name | Value | Description |
| --- | --- | --- |
| msgId | 1105 | Real-time event message ID |
| result | 0 | Server -> Client (0: Fixed) |
| body |  |  |
| TerminalID | 8 | Terminal ID |

WSCmdGroupSyncNotify (msgId: 1106)

: When adding the group in the group management page

: Received data :

{ msgId: 1106, result: 0, body: "{"SyncType":1,"GroupInfo":{"GroupID":2,"Parent":1, "Name":"test1", "Description":""}}”}

(When having the data below)

| Name | Value | Description |
| --- | --- | --- |
| msgId | 1106 | Real-time event message ID |
| result | 0 | Server -> Client (0: Fixed) |
| body |  |  |
| SyncType | 1 | Type 1 : insert 2 : update 3 : delete |
| GroupInfo |  |  |
| - GroupID | 2 | Group ID |
| - Parent | 1 | The ID of parent group |
| - Name | test1 | Group name |
| - Description |  | Description |

WSCmdPositionSyncNotify (msgId: 1107)

: When clicking [Apply] button after adding the position in position management page : Received data :

{ msgId: 1107, result: 0, body: "{"SyncType":1,"PositionInfo":{"PositionID":3,"Name":"test456"}}"} (When having the data below)

| Name | Value | Description |
| --- | --- | --- |
| msgId | 1107 | Real-time event message ID |
| result | 0 | Server -> Client (0: Fixed) |
| body |  |  |

| SyncType | 1 | Type 1 : insert 2 : update 3 : delete |
| --- | --- | --- |
| PositionInfo |  |  |
| - PositionID | 3 | Position ID |
| - Name | test456 | Position name |

WSCmdTerminalSyncNotify (msgId: 1108)

: When adding the terminal in terminal management page

: Received data :

{msgId: 1108, result: 0, body: “{"SyncType": 1,"TerminalInfo": {"ID": 10, "Name": "10", "GroupCode": 0,"Status": 16,"Type": 0,"FuncType": 0,"IPAddress": "","MacAddress": "", "Version": "","RemoteDoor": 0,"UTCIndex": 0,"Description": ""}}”}

(When having the data below)

| Name | Value | Description |
| --- | --- | --- |
| msgId | 1108 | Real-time event message ID |
| result | 0 | Server -> Client (0: Fixed) |
| body |  |  |
| SyncType | 1 | Type 1 : insert // add 2 : update 3 : delete 4 : insert // unRegist |
| TerminalInfo |  |  |
| - ID | 10 | Terminal ID |
| - Name | 10 | Terminal name |
| - GroupCode | 0 | Group code |
| - Status | 16 | Terminal status value, No use |
| - Type | 0 | Terminal type |
| - FuncType | 0 | Function key, No use |
| - IPAddress |  | IP address |
| - MacAddress |  | Mac address |
| - Version |  | Terminal version |
| - RemoteDoor | 0 | Remote door, No use |
| - UTCIndex | 0 | UTC information, No use |
| - Description |  | Description, No use |

WSCmdAccessGroupSyncNotify (msgId: 1109)

: Add the access group in the access group management page

-> The event occurs when clicking [Save] button after setting

: Received data :

{ msgId: 1109, result: 0, body: "{"SyncType":1,"AccessGroupInfo":{"ID":3, "Name":"Access group 3","TimezoneID":0}}"}

(When having the data below)

| Name | Value | Description |
| --- | --- | --- |
| msgId | 1109 | Real-time event message ID |
| result | 0 | Server -> Client (0: Fixed) |
| body |  |  |
| SyncType | 1 | Type 1 : insert 2 : update 3 : delete |
| AccessGroupInfo |  |  |
| - ID | 3 | Group ID |
| - Name | Access group 3 | Group name |
| - TimezoneID | 0 | Time zone ID |

WSCmdTimezoneSyncNotify (msgId: 1110)

: When clicking [Save] button after adding the time zone in time zone  management page

: Received data :

{msgId: 1110, result: 0, body: “{"SyncType": 2,"TimezoneInfo": {"TimezoneID": 1, "Name": "New","HolidayID": 2,"Holiday": 1,"Sunday": 1,"Monday": 1,"Tuesday": 1, "Wednesday": 1,"Thursday": 1,"Friday": 1,"Saturday": 1}}”}

(When having the data below)

| Name | Value | Description |
| --- | --- | --- |
| msgId | 1110 | Real-time event message ID |
| result | 0 | Server -> Client (0: Fixed) |
| body |  |  |
| SyncType | 2 | Type 1 : insert 2 : update 3 : delete |
| TimezoneInfo |  |  |
| - TimezoneID | 1 | Time zone ID |
| - Name | New | Time zone name |
| - HolidayID | 2 | Holiday ID |
| - Holiday | 1 | Holiday (1 : Fixed) |
| - Sunday | 1 | Sunday (1 : Fixed) |
| - Monday | 1 | Monday (1 : Fixed) |
| - Tuesday | 1 | Tuesday (1 : Fixed) |
| - Wednesday | 1 | Wednesday (1 : Fixed) |
| - Thursday | 1 | Thursday (1 : Fixed) |
| - Friday | 1 | Friday (1 : Fixed) |
| - Saturday | 1 | Saturday (1 : Fixed) |

WSCmdPrivilegeSyncNotify (msgId: 1112)

: When clicking [Save] button after adding the privilege in admin management page

: Received data :

{msgId: 1112, result: 0, body:

"{"SyncType":1,"PrivilegeInfo":{"PrivilegeID":1237,"Name":"test123","Description":"test1","Monitoring ":{"Permit":0,"Client":0,"TerminalStatus":0,"Auth":0,"Event":0},"Terminal":{"Permit":1,"Regist":1,"Upd ate":1,"Delete":1,"FW":0,"Option":0,"SetManager":0,"UserFile":0},"User":{"Permit":1,"Regist":1,"Upd ate":1,"Delete":1,"TerminalUser":0,"RegistAdmin":0},"Group":{"Permit":1,"Regist":1,"Update":1,"Dele te":1,"User":0},"Guest":{"Permit":0,"Regist":0,"Update":0,"Delete":0},"Blacklist":{"Permit":0,"Change ":0,"Release":0,"Update":0,"Delete":0},"AccessControl":{"Permit":0,"Set":0},"Map":{"Permit":0,"Set":

0},"TNA":{"Permit":0,"Set":0,"Special":0,"Manage":0,"OutState":0,"OutExcRecord":0,"Summary":0,"S endResult":0,"DeleteResult":0},"Log":{"Permit":0,"Delete":0,"BackupRestore":0},"Meal":{"Permit":0," DeleteResult":0,"ViewGroup":0,"ViewPersonal":0,"Set":0},"Option":{"Permit":0,"Local":0,"Server":0,"P assword":0,"Mail":0,"TNA":0},"GroupInfo":{"GroupID":0,"Parent":0,"Name":"","Description":""}}}"}

( When having the data below)

| Name | Value | Description |
| --- | --- | --- |
| msgId | 1112 | Real-time event message ID |
| result | 0 | Server -> Client (0: Fixed) |
| body |  |  |
| SyncType | 1 | Type 1 : insert 2 : update 3 : delete |
| PrivilegeInfo |  |  |
| - PrivilegeID | 1237 | Privilege ID |
| - Name | test123 | Privilege name |
| - Description | test1 | Description |
| - Monitoring |  |  |
| - Permit | 0 | Whether to permit or not (If checked, 1 : Fixed) |
| - Client | 0 | Client |
| - TerminalStatus | 0 | Terminal status |
| - Auth | 0 | Authentication log |
| - Event | 0 | Event log |
| - Terminal |  |  |
| - Permit | 1 | (If checked, 1 : Fixed) |
| - Regist | 1 | Registration |
| - Update | 1 | Modification |
| - Delete | 1 | Deletion |
| - FW | 0 | Apply the FW. |
| - Option | 0 | Option setting |
| - SetManager | 0 | Set the admin. |
| - UserFile | 0 | User file |
| - User |  |  |
| - Permit | 1 | (If checked, 1 : Fixed) |
| - Regist | 1 | Registration |
| - Update | 1 | Modification |
| - Delete | 1 | Deletion |
| - TerminalUser | 0 | Terminal user |
| - RegistAdmin | 0 | User file |
| - Group |  |  |
| - Permit | 1 | (If checked, 1 : Fixed) |
| - Regist | 1 | Registration |
| - Update | 1 | Modification |
| - Delete | 1 | Deletion |
| - User | 0 | User |
| - Guest |  |  |
| - Permit | 0 | (If checked, 1 : Fixed) |
| - Regist | 0 | Registration |
| - Update | 0 | Modification |
| - Delete | 0 | Deletion |
| - Blacklist |  |  |
| - Permit | 0 | (If checked, 1 : Fixed) |

| - Change | 0 | Modification |
| --- | --- | --- |
| - Release | 0 | Release |
| - Update | 0 | Change |
| - Delete | 0 | Deletion |
| - AccessControl |  |  |
| - Permit | 0 | (If checked, 1 : Fixed) |
| - Set | 0 | Registration |
| - Map |  |  |
| - Permit | 0 | (If checked, 1 : Fixed) |
| - Set | 0 | Set the draft. |
| - TNA |  |  |
| - Permit | 0 | (If checked, 1 : Fixed) |
| - Set | 0 | Set the TNA. |
| - Special | 0 | Set the special shift. |
| - Manage | 0 | TNA management |
| - OutState | 0 | Search the working status. |
| - OutExcRecord | 0 | Search the exclusion report. |
| - Summary | 0 | Manage the TNA summary. |
| - SendResult | 0 | Send the TNA result. |
| - DeleteResult | 0 | Delete the TNA result. |
| - Log |  |  |
| - Permit | 0 | (If checked, 1 : Fixed) |
| - Delete | 0 | Deletion |
| - BackupRestore | 0 | Restoration |
| - Meal |  |  |
| - Permit | 0 | (If checked, 1 : Fixed) |
| - DeleteResult | 0 | Search the meal report. |
| - ViewGroup | 0 | Search the meal by group. |
| - ViewPersonal | 0 | Search the meal by individual. |
| - Set | 0 | Setting |
| - Option |  |  |
| - Permit | 0 | (If checked, 1 : Fixed) |
| - Local | 0 | Area |
| - Server | 0 | Server |
| - Password | 0 | Password |
| - Mail | 0 | Mail |
| - TNA | 0 | TNA |
| - GroupInfo |  |  |
| - GroupID | 0 | Group ID |
| - Parent | 0 | The parent group’s ID |
| - Name |  | Group name |
| - Description |  | Description |

WSCmdUserMessageSyncNotify (msgId: 1113)

: When adding the user message in user message management page

: Received data:

{ msgId: 1113, result: 0, body: "{"SyncType":1,"UserMessageInfo":{"MessageID":2, "Message":"test1234"}}"}

(When having the data below)

| Name | Value | Description |
| --- | --- | --- |
| msgId | 1113 | Real-time event message ID |
| result | 0 | Server -> Client (0: Fixed) |
| body |  |  |
| SyncType | 1 | Type 1 : insert 2 : update 3 : delete |
| UserMessageInfo |  |  |
| - MessageID | 2 | Message ID |
| - Message | test1234 | Message contents |

WSCmdTerminalLogUploadNotify (msgId: 1120)

: When clicking [Import] after checking the terminal in Terminal log page

: Received data :

{ msgId: 1120, result: 0, body: "{"terminalID":8, "total":2397, "offset":2384, "count":13}"}

| Name | Value | Description |
| --- | --- | --- |
| msgId | 1120 | Real-time event message ID |
| result | 0 | Server -> Client (0: Fixed) |
| body |  |  |
| terminalID | 8 | Terminal ID |
| total | 2397 | The number of total logs. |
| offset | 2384 | The number of current log (It doesn’t import the number of total log of terminal but does the number of count in order. |
| count | 13 | The number of log to import at once |

Table 1 – Event code

| Event code | Event name | Event code | Event name |
| --- | --- | --- | --- |
| 65537 | Terminal Disconnected | 196614 | Panic Sensor End |
| 65538 | Terminal Connected | 196615 | Crisis Sensor Start |
| 65539 | Terminal Locked | 196616 | Crisis Sensor End |
| 65540 | Terminal Unlocked | 196617 | Blacklist authentication trial |
| 65541 | Terminal Tamper | 196624 | Threat |
| 65542 | Terminal Attached | 196625 | System error |
| 65543 | Terminal Lockdown | 196626 | Door warning |
| 131073 | Door Open | 196627 | Door 2 open warning |
| 131074 | Door Close | 196628 | Door 2 emergency warning |
| 131075 | Door Unlock | 196629 | Release the door 2 open warning. |
| 131076 | Door Locked | 196630 | Fire Sensor Start |
| 131077 | Door Forced | 196631 | Panic Sensor Start |
| 131078 | Open too long | 196632 | Panic Sensor End |
| 131079 | Door Restored | 196633 | Fire Sensor End |
| 131080 | Lock Error | 196640 | FP device error |
| 131081 | Not Monitoring | 196641 | DB Error |
| 131083 | Door open | 196642 | RTC Error |
| 131084 | Remove the door open. | 196643 | Touch Error |
| 131088 | Operate the door open. | 262145 | External Sensor 1 Start |
| 131089 | Operate to remove the door lock. | 262146 | External Sensor 1 End |
| 131090 | Operate the door lock. | 262147 | External Sensor 2 Start |
| 131091 | Door change | 262148 | External Sensor 2 End |
| 131092 | Open button pressed in Door 1. | 262149 | External Sensor 3 Start |
| 131093 | Door 2 Open | 262150 | External Sensor 3 End |
| 131094 | Door 2 Closed | 262151 | External Sensor 4 Start |
| 131095 | Open button pressed in Door 2. | 262152 | External Sensor 4 End |
| 131096 | Remove the open too long. | 327681 | Change the FP device setting. |
| 196609 | Arm | 327682 | Change the UI setting. |
| 196610 | Disarm | 327683 | Change the system. |
| 196611 | Fire Sensor Start | 327684 | Change the time. |
| 196612 | Fire Sensor End | 327685 | Change the fixed setting. |
| 196613 | Panic Sensor Start | 327686 | Change the all options. |

Table 2 – Alpeta Terminal Status

| Status name | SHIFT | Description |
| --- | --- | --- |
| TerminalStatusConnect | 1 << 0 | Connection - 0: Disconnection, 1: Connection |
| TerminalStatusRegist | 1 << 4 | Registration status - 0: Unregistered, 1: Registered |
| TerminalStatusIDConflict | 1 << 5 | ID conflict - 0: Normal 1: ID conflict |
| TerminalStatusInvalidType | 1 << 6 | The registered terminal type is different. - 0: Normal, 1: Different |
| TerminalStatusSyncUserIDLen | 1 << 15 | The length of user ID |

|  |  | - 0: Matched, 1: Different |
| --- | --- | --- |
| TerminalStatusSyncMacAddres | 1 << 16 | Mac Address - 0: Normal, 1: Different |
| TerminalStatusSyncUserCount | 1 << 17 | The number of user - 0: Normal, 1: Different |
| TerminalStatusSyncFWVersion | 1 << 18 | FW version - 0: Normal, 1: Different |
| TerminalStatusDoor | 1 << 7 | Door - 0: Closed, 1: Open |
| TerminalStatusDoorEmergency | 1 << 8 | Door Forced - 0: Normal, 1: Forced |
| TerminalStatusDoorOpenWarn | 1 << 9 | Open too long - 0: Normal, 1: Open too long |
| TerminalStatusDoorLookState | 1 << 10 | Status - 0: Locked, 1: Open |
| TerminalStatusDoorLockWorking | 1 << 11 | Operation - 0: Normal, 1: Abnormal |
| TerminalStatusDoorNotMonitoring | 1 << 23 | When door status cannot be monitored |
| TerminalStatusDoorOpenState | 1 << 24 | The status of door open (By the command of door open) |
| TerminalStatusLock | 1 << 1 | Lock - 0: Open, 1: Lock |
| TerminalStatusLockForce | 1 << 2 | Lockdown - 0: Normal, 1:Lockdown |
| TerminalStatusCover | 1 << 3 | Cover - 0: Attached, 1: Terminal tamper |
| TerminalStatusWarnFire | 1 << 12 | Fire - 0: Normal, 1: Fire |
| TerminalStatusWarnPanic | 1 << 13 | Panic - 0: Normal, 1: Panic |
| TerminalStatusWarnCricis | 1 << 14 | Crisis - 0: Normal, 1: Crisis |

Connection status Door : Door lock Warning